| PARTS |  |  |  |
| --- | --- | --- | --- |
| ID | Description | QTY |  |
| 1 | PVC Pipe, ½”, 25 mm length | 7 |  |
| 2 | PVC Pipe, ½”, 30 mm length | 3 |  |
| 3 | PVC Pipe, ½”, 60 mm - xx mm length | 2 |  |
| 4 | PVC Union, ½” Soc x ½” Soc | 1 |  |
| 5 | PVC Elbow, ½” Soc x ½” Soc | 6 |  |
| 6 | PVC Tee, ½” Soc x ½” Soc x ½” Soc | 3 |  |
| 7 | PVC Adapter, ½” NPT x ½” Female Soc Adapter | 1 |  |
| 8 | C-Clamp, 2-1/2” | 2 |  |
| TOOLS AND SUPPLIES |  |  |
| Description |  |  |
| PVC Cutters OR Hacksaw |  |  |
| PVC Cement (Optional) |  |  |