| PARTS |  |  |
| --- | --- | --- |
| 2 1 3 1 2 2 3 1 1 2 1 1 |  |  |
| ID | Description | QTY |
| 1 | PVC Elbow 90 deg, ½” | 6 |
| 2 | PVC Pipe, ½”, 30 mm length | 4 |
| 3 | PVC Pipe, ½”, 30 mm OR 70 mm for use with Quick Clamp | 2 |

TOOLS AND SUPPLIES

Description

PVC Cutters OR Hacksaw

PVC Cement (Optional)

| 01. | Use the PVC cutters or hacksaw to cut the PVC pipe to length. | 02. | Insert two short PVC sections into an elbow. |
| --- | --- | --- | --- |
| 03. | Insert an elbow onto each pipe section. | 04. | Repeat Steps 2 and 3 to create a second side. |
| 05. | Insert longer PVC section into elbows. | 06. | Connect second side. |
| 07. | (Optional) Use pencil to mark lines across joints. | 08. | (Optional) Disassemble and reassemble with PVC cement. Erase pencil marks. |